CAMERA MANAGEMENT SYSTEM

User Manual

Camera Management System - Software version 1.0.0

| Field | Value |
| --- | --- |
| Document ID | CMS-UM-001 |
| Document Title | User Manual |
| Product | Camera Management System (CMS) |
| Software Version | 1.0.0 |
| Version | 1.0 |
| Issue Date | 2026-09-21 |
| Status | Issued for Review |

## Revision History

| Version | Date | Author | Summary of Change |
| --- | --- | --- | --- |
| 1.0 | 2026-09-21 | Engineering | First issue, covering software version 1.0.0. |

# Contents

Revision History	1

Contents	2

1. About This Manual	5

1.1 Who This Manual Is For	5

1.2 How This Manual Is Organised	5

1.3 Conventions	5

2. Getting Started	6

2.1 What You Need	6

2.2 Installing	6

2.3 Starting the Application	6

2.4 Signing In	6

2.5 Change the Default Password	6

2.6 Signing Out and Closing	7

3. Finding Your Way Around	8

3.1 The Window	8

3.2 The Alert Bell	8

3.3 Why You May See Fewer Screens Than a Colleague	8

3.4 Keyboard Shortcuts	8

4. Setting Up Cameras	9

4.1 Finding Cameras Automatically	9

4.2 Adding a Camera by Hand	9

4.3 What Each Field Means	9

4.4 Changing or Removing a Camera	10

5. Watching Live Video	11

5.1 Live View	11

5.2 Camera Status	11

5.3 Viewing One Camera Full Screen	11

5.4 Camera List	11

6. Aiming a Moveable Camera	12

6.1 Moving the Camera	12

6.2 Presets	12

7. Recording	13

7.1 Turning Recording On	13

7.2 Where Recordings Are Kept	13

7.3 Managing Disk Space	13

8. Playing Back Recordings	13

9. Detecting People and Vehicles	14

9.1 What You Need	14

9.2 Turning Detection On	14

9.3 Reading the Screen	14

9.4 Choosing What to Look For	14

9.5 Adjusting Sensitivity	14

10. Recognising Faces	15

10.1 What You Need	15

10.2 Registering a Person	15

10.3 Getting Good Results	15

10.4 Registering Several People at Once	15

10.5 Watching Recognition	16

10.6 If Recognition Is Wrong	16

10.7 Changing or Removing a Registered Person	16

11. The Event Log	17

11.1 What Is Recorded	17

11.2 Finding an Event	17

11.3 Exporting	17

11.4 Jumping to the Camera	17

12. Managing Accounts	18

12.1 Roles	18

12.2 Adding an Account	18

12.3 Changing What Someone Can See	18

12.4 Changing a Password	18

12.5 Things the System Will Not Let You Do	18

13. Settings	19

13.1 General	19

13.2 Network	19

13.3 Storage	19

13.4 AI Models	19

13.5 Restoring Defaults	20

14. System Information	20

15. Solving Problems	21

15.1 Starting and Signing In	21

15.2 Cameras and Video	21

15.3 Detection and Face Recognition	21

15.4 Recording and Playback	22

15.5 Events	22

15.6 Before Contacting Support	22

16. Reference	23

16.1 Default Account	23

16.2 Where Things Are Kept	23

16.3 Permissions	23

16.4 Glossary	23

# 1. About This Manual

## 1.1 Who This Manual Is For

This manual is for the people who use the Camera Management System day to day: operators who watch live video and respond to events, and administrators who install the system, add cameras and manage accounts.

No programming knowledge is assumed. Where a task can only be done by an administrator, the manual says so.

## 1.2 How This Manual Is Organised

| If you want to | Read |
| --- | --- |
| Install and start the system for the first time | Section 2 |
| Understand what you are looking at | Section 3 |
| Add cameras | Section 4 |
| Watch live video | Section 5 |
| Aim a moveable camera | Section 6 |
| Record video and play it back | Sections 7 and 8 |
| Detect people and vehicles | Section 9 |
| Register and recognise faces | Section 10 |
| Review what has happened | Section 11 |
| Manage accounts | Section 12 |
| Change how the system behaves | Section 13 |
| Fix a problem | Section 15 |

## 1.3 Conventions

| Convention | Meaning |
| --- | --- |
| Screen name | A screen in the navigation list at the left, for example Live View. |
| Button name | Something you click, for example Save. |
| Note | Extra information that helps, but is not essential. |
| Important | Something that can lose data, or that you cannot undo. |

# 2. Getting Started

## 2.1 What You Need

| Item | Requirement |
| --- | --- |
| Computer | Windows 10 or Windows 11, 64-bit. Four processor cores or more if you plan to use detection or face recognition. |
| Memory | 8 GB minimum. 16 GB if several cameras will run analytics. |
| Disk | Enough free space for the recordings you intend to keep. |
| Network | The computer must be on the same network as the cameras. |
| Cameras | IP cameras that support ONVIF, RTSP, or both. |
| Internet | Not required. The system never contacts the internet. |

## 2.2 Installing

There is no installer. Copy the application folder to the computer and run it from there.

- Copy the whole folder to a location you can write to, for example C:\CameraManagementSystem.

- Double-click CameraManagementSystem.exe.

Note   The system stores its settings and its camera list in your Windows user profile, not in the application folder. Copying the folder to another computer does not carry your cameras with it.

## 2.3 Starting the Application

A start-up screen appears while the system opens its database and loads the detection models. This normally takes a few seconds. If the detection models are not installed, start-up still completes; only the detection and face features are unavailable.

## 2.4 Signing In

The sign-in window appears next. The first time the system runs, one account exists:

| Username | Password | Role |
| --- | --- | --- |
| admin | admin1234 | Administrator |

Type the username and password and click Sign In. Press Escape to close the application instead.

If the details are wrong, the system reports that the username or password is incorrect. It deliberately does not say which of the two was wrong, so that the screen cannot be used to discover which accounts exist.

## 2.5 Change the Default Password

Important   The default password is published in this manual, so everyone who has this manual can sign in to a system that still uses it. Change it before the system is put into service.

To change it:

- Sign in as admin.

- Open User Management.

- Click the edit icon on the admin row.

- Type a new password, type it again to confirm, and click Save.

Note   Only an administrator can change a password, including their own, and it is done from User Management. There is no self-service password change on the sign-in screen.

Important   There is no password recovery. If every administrator password is lost, the only way back in is to delete the database file named in section 16.2, which also deletes every camera, account, registered face and event.

## 2.6 Signing Out and Closing

| To | Do this |
| --- | --- |
| Sign out | Click your account name in the header, then Sign Out, then confirm. Video and recording stop and the sign-in window returns. |
| Close the application | Click the close button at the top right, then confirm. |

# 3. Finding Your Way Around

## 3.1 The Window

| Area | What it is |
| --- | --- |
| Header, along the top | The product name at the left. At the right, your account name, the alert bell, the clock, and the window buttons. |
| Navigation list, at the left | The screens you can open. Click one to go to it. |
| Main area | The screen you have opened. Its name appears at the top left, and any buttons for that screen at the top right. |

## 3.2 The Alert Bell

The bell in the header shows a number when something has happened that may need attention, such as a camera going offline. Click it to open the Event Log; the number clears.

Note   Routine activity, such as a person being detected, does not raise the count. Only warnings and alarms do.

## 3.3 Why You May See Fewer Screens Than a Colleague

The navigation list only shows the screens your account is permitted to use. If a screen described in this manual is not in your list, your account does not have permission for it. Ask an administrator; section 12 explains how permissions are granted.

## 3.4 Keyboard Shortcuts

| Key | What it does |
| --- | --- |
| F5 | Refreshes the screen you are on. |
| F11 | Makes the window full screen, or returns it to its normal size. |
| Escape | Closes the window you are in, or leaves a full-screen camera view. |
| Enter | Accepts the main button in a dialog. |

# 4. Setting Up Cameras

Adding cameras requires the Device Management permission. Open Device Management from the navigation list.

## 4.1 Finding Cameras Automatically

This is the easiest way to add a camera, and works with any camera that supports ONVIF.

- Click Scan.

- Wait a few seconds while the system looks for cameras on your network.

- Select a camera from the list and confirm.

- The address is filled in for you. Type the camera's username and password.

- Click Save.

Note   If the scan finds nothing, the cameras may be on a different network, or your network may block the discovery message. Add the camera by hand instead, as described next.

## 4.2 Adding a Camera by Hand

- Click Add.

- Fill in the fields described in section 4.3.

- Click Test Connection to check the details before saving.

- Click Save.

## 4.3 What Each Field Means

| Field | What to enter |
| --- | --- |
| Name | A name that means something to you, for example Front Entrance. |
| Channel | A number that identifies this camera. Each camera needs its own. |
| IP Address | The camera's address on the network. |
| RTSP Port | The port the video comes from. Usually 554. |
| ONVIF Port | The port used for camera control. Usually 80. |
| Username and Password | The camera's own login, not your CMS login. |
| Protocol | RTSP, ONVIF or HTTP. Choose ONVIF if the camera supports it. |
| Type | IP Camera, PTZ Camera, NVR or USB Camera. |
| Object Detection | Tick to look for people and vehicles on this camera. |
| Face Recognition | Tick to look for faces on this camera. |
| Recording | Tick to record this camera continuously. |
| Enabled | Untick to keep the camera in the list but stop connecting to it. |

Note   Detection and face recognition cost processing time on every camera you enable them for. Turn them on only where you need them.

## 4.4 Changing or Removing a Camera

| To | Do this |
| --- | --- |
| Change a camera | Click the edit icon on its row, change what you need, and click Save. |
| Stop using a camera temporarily | Edit it and untick Enabled. |
| Remove a camera | Click the delete icon on its row and confirm. |

Note   Removing a camera does not delete recordings already made from it.

# 5. Watching Live Video

## 5.1 Live View

Open Live View. Cameras connect automatically and appear as tiles. Use the buttons at the top right to choose how many tiles are shown at once, or pick a single camera from the selector.

## 5.2 Camera Status

| Shown as | Meaning |
| --- | --- |
| Online, green | Video is arriving normally. |
| Connecting, amber | The system is trying to reach the camera. |
| Offline, red | The camera cannot be reached. The system keeps retrying. |
| Error, red | The camera answered but the video could not be read. |

Note   A camera that drops out reconnects on its own. You do not need to restart anything. One camera failing never affects the others.

## 5.3 Viewing One Camera Full Screen

Click a tile to open that camera full screen. Press Escape to go back.

## 5.4 Camera List

Camera List shows every channel as a wall of tiles, with buttons to choose two, four or six across. It is useful for watching everything at once; Live View is better for watching a few closely.

# 6. Aiming a Moveable Camera

Cameras that can pan, tilt and zoom are called PTZ cameras. This needs the PTZ Control permission.

## 6.1 Moving the Camera

- Open PTZ Control.

- Choose a camera from the selector at the top right. Only cameras that can move are listed.

- Press and hold an arrow to move the camera. Release to stop.

- Use the zoom controls to zoom in and out.

- Use the speed control to make movement faster or slower.

Note   If the camera keeps moving after you let go, it is the camera that is slow to respond rather than the system; the stop command is sent as soon as you release.

## 6.2 Presets

A preset is a saved position you can return to with one click.

| To | Do this |
| --- | --- |
| Go to a preset | Select it in the preset list. |
| Save a new preset | Aim the camera, click Set Preset, and give it a name. |
| Remove a preset | Select it and click Remove. |

Note   Presets are stored in the camera itself, not in this system, so they are shared with anything else that controls that camera.

# 7. Recording

## 7.1 Turning Recording On

Edit a camera in Device Management and tick Recording. The camera records continuously from then on, in segments of about ten minutes.

## 7.2 Where Recordings Are Kept

Recordings are written to the storage folder set in Settings, under Storage. The default location is given in section 16.2. To change it, open Settings, choose Storage, click the browse button beside Storage Folder, pick a folder, and click Save.

## 7.3 Managing Disk Space

| Setting | What it does |
| --- | --- |
| Retention Days | How long recordings are kept. Older recordings are deleted automatically. Default 30 days. |
| Maximum Storage | The most disk space recordings may use. Default 512 GB. |
| Overwrite When Full | When ticked, the oldest recordings are deleted to make room. When unticked, recording stops once the limit is reached. |

Important   Recordings deleted by the retention or storage rules cannot be recovered. Check these settings before leaving the system to run unattended.

The Dashboard shows how much space recordings currently use.

# 8. Playing Back Recordings

- Open Playback.

- Choose a camera and a date at the top right.

- The bar across the screen shows the whole day. Shaded parts are recorded; highlighted parts contain events.

- Click anywhere on the bar to jump to that time.

- Use play and pause to control the video.

- The list beside the video shows events from that day. Selecting one jumps the video to it.

Note   If the bar is empty for a day you expected, check that the camera had Recording ticked at the time, and that the retention period has not already removed it.

# 9. Detecting People and Vehicles

## 9.1 What You Need

Object detection needs a detection model file, which is installed separately because of its size and licensing. If it is not installed, the Object Detection screen says so and everything else continues to work.

To install one, obtain a YOLO detection model in ONNX form, put it in the Models folder inside the application folder, and make sure the file name matches the one in Settings under AI Models. Then open Settings and click Reload Models.

## 9.2 Turning Detection On

- Edit the camera in Device Management and tick Object Detection.

- Open Object Detection and choose the camera at the top right.

## 9.3 Reading the Screen

| Part | What it shows |
| --- | --- |
| Video | A box around each thing found, labelled with what it is and how confident the system is. |
| Detection Settings | The model in use, the confidence threshold, how often frames are checked, and which classes are being looked for. |
| Detection Count | How many of each kind are visible right now. |

## 9.4 Choosing What to Look For

In Detection Settings, tick only the classes you care about. Anything unticked is ignored and raises no events. By default the system looks for people, cars, bicycles, motorcycles, buses, trucks, dogs and cats.

## 9.5 Adjusting Sensitivity

| Setting | Effect |
| --- | --- |
| Confidence threshold | Higher means fewer, more certain detections. Lower means more detections and more false ones. Default 0.50. |
| Detection interval | How often a frame is checked, in milliseconds. Larger numbers use less processing power but react more slowly. Default 200. |

Note   Events are not raised every time something is seen. Once an object of a given kind has been reported for a camera, it is not reported again for twenty seconds, so a parked car does not fill the log.

# 10. Recognising Faces

## 10.1 What You Need

Face recognition uses two model files: one that finds faces in the picture, and one that turns a face into a numeric fingerprint for comparison. Both are installed separately.

Put both files in the Models folder inside the application folder, using the names shown in Settings under AI Models, then open Settings and click Reload Models. The message tells you whether each one loaded.

Note   A file in the Models folder next to the README explains which model files are known to work. If you are supplied with models by your installer, use those.

## 10.2 Registering a Person

- Open Face Database.

- Click Add Face.

- Click Photo to choose a picture from disk, or choose a camera and click Capture to take the picture from live video.

- Type the person's name.

- Choose a group: Employee, Visitor, Blacklist or Unknown.

- Add a note if you want one.

- Click Save.

If the picture contains more than one face, the largest is used. If no face can be found, the system says so and nothing is saved.

## 10.3 Getting Good Results

The quality of the registration photograph decides how well the person is recognised afterwards. For the best results:

- Use a photograph taken face-on, not at an angle.

- Make sure the face is evenly lit, with no heavy shadow across it.

- Use a picture where the face is reasonably large and in focus.

- Avoid sunglasses, hats pulled low, or anything covering the face.

- Where possible, take the registration picture with the same camera that will do the recognising.

## 10.4 Registering Several People at Once

- Put one photograph per person in a folder, and name each file after the person.

- Open Face Database and click Import Folder.

- Choose the folder.

Each person is registered under their file name, in the Employee group. When finished, the system reports how many were registered and lists any pictures in which no face could be found.

## 10.5 Watching Recognition

Tick Face Recognition on a camera in Device Management, then open Face Recognition and choose that camera. Faces are boxed and labelled with the person's name and a percentage, or with Unknown if nobody registered is a close enough match.

The Latest Match panel shows the most recent person recognised. The list below shows recent face events.

## 10.6 If Recognition Is Wrong

| What you see | What to do |
| --- | --- |
| People you have registered show as Unknown | Register them again from a clearer, face-on photograph. If it still happens, lower the face match threshold in Settings a little at a time. |
| Strangers are matched to a registered name | Raise the face match threshold in Settings a little at a time. |
| Almost everyone matches almost everyone | This is not a threshold problem. The recognition model is probably not one the system supports. See section 15. |
| No faces are boxed at all, even close up | The face-finding model may not be one the system supports. See section 15. |

Note   The face match threshold is in Settings under AI Models. It is the percentage a face must reach to count as a match. The default of 0.68 suits the recommended models. Change it in small steps and check the result each time.

## 10.7 Changing or Removing a Registered Person

| To | Do this |
| --- | --- |
| Change a name, group or note | Click the edit icon, change the details and click Save. The stored face fingerprint is not altered. |
| Replace the photograph | Click the edit icon, choose a new photograph and click Save. A new fingerprint is made from it. |
| Stop recognising someone temporarily | Edit them and untick Active. |
| Remove someone | Click the delete icon and confirm. |

# 11. The Event Log

## 11.1 What Is Recorded

| Event | When it happens |
| --- | --- |
| Object Detected | A class you are looking for appeared on a camera. |
| Face Recognized | A registered person was recognised. |
| Camera Offline | A camera stopped responding. |
| Camera Online | A camera came back. |
| Recording | A recording started, finished, or could not be written. |
| System | Sign-in, sign-out, configuration changes and faults. |

Each event is marked Info, Warning or Critical. Only Warning and Critical raise the count on the alert bell.

## 11.2 Finding an Event

Open Event Log. The controls at the top right let you narrow the list by kind, by camera, and by a date range, and search the text of the messages. The filters work together.

## 11.3 Exporting

Set the filters so the list shows what you want, then click Export. The system saves a CSV file, which opens in a spreadsheet, containing exactly the rows shown.

## 11.4 Jumping to the Camera

Double-click an event that came from a camera to open that camera.

# 12. Managing Accounts

This needs the User Management permission. Open User Management.

## 12.1 Roles

| Role | Can do |
| --- | --- |
| Administrator | Everything, including managing accounts and settings. An administrator always has every permission and the list cannot be changed. |
| Operator | Watch live video, play back recordings, use detection and face recognition, and aim PTZ cameras. |
| Viewer | Watch live video and play back recordings. |

## 12.2 Adding an Account

- Click Add User.

- Type a username. It must not already be in use.

- Type a password, and type it again to confirm. It must be at least six characters.

- Choose a role.

- Click Save.

## 12.3 Changing What Someone Can See

- Select the person in the list.

- Tick or untick the screens in the Permissions panel at the right.

- Click Save.

Note   A change takes effect the next time that person signs in.

## 12.4 Changing a Password

Click the edit icon on the account, type the new password twice, and click Save. Leaving the password fields empty keeps the existing password.

## 12.5 Things the System Will Not Let You Do

- You cannot delete the account you are signed in with.

- You cannot delete the last administrator that is still active. Create another one first.

- You cannot create two accounts with the same username, even with different capitals.

# 13. Settings

This needs the System Settings permission. Open Settings and choose a group at the left. Changes are not applied until you click Save.

## 13.1 General

| Setting | What it does |
| --- | --- |
| System Name | A name for this installation. |
| Language | Reserved for future use. All text is currently English whatever you choose. |
| Time Zone | The time zone used to display times. |
| Date Format | How dates are written. |
| Auto Logout | Sign out automatically after a period of no activity. |
| Start automatically with Windows | Start the system when the computer starts. |

## 13.2 Network

| Setting | What it does |
| --- | --- |
| ONVIF Discovery Timeout | How long Scan looks for cameras. Increase it on a large or slow network. |
| RTSP Connect Timeout | How long to wait for a camera before treating it as offline. |
| Prefer TCP Transport | Leave ticked. TCP gives a steadier picture than the alternative. |

## 13.3 Storage

Covered in section 7.3.

## 13.4 AI Models

| Setting | What it does |
| --- | --- |
| Object Model Path | Where the detection model file is. A plain file name means the Models folder inside the application folder; a full path may be used instead. |
| Face Detector Path | Where the face-finding model file is. |
| Face Recognition Path | Where the face-fingerprint model file is. |
| Detection Interval | How often frames are checked, in milliseconds. |
| Face Match Threshold | How close a face must be to count as a match. See section 10.6. |
| Use GPU acceleration | Uses the graphics card if a supported one is present. The system falls back to the processor by itself if not. |
| Reload Models | Applies the paths above without restarting, and reports what happened to each model. |

## 13.5 Restoring Defaults

Restore Defaults returns every setting to the value it had when the system was installed. Your cameras, accounts, registered faces and recordings are not affected.

# 14. System Information

System Information shows what version is running, how the computer is loaded, the network settings in use, and the state of the detection models. It is the first place to look when something is not working, and the details it shows are what a support engineer will ask for.

Open Data Folder opens the folder holding the database, which is useful when taking a backup.

Note   Check for Update does not contact anything. The system is designed to run without internet access, so updates are applied by replacing the application folder.

# 15. Solving Problems

## 15.1 Starting and Signing In

| Problem | What to check |
| --- | --- |
| The application will not start and reports that the database cannot be opened | Another copy may already be running; check for it before starting another. Otherwise your Windows profile may be read-only or full. |
| The password is not accepted | Check the capitals. If this is a new installation, the account is admin with password admin1234. |
| Nobody can sign in as an administrator | There is no recovery. See the warning in section 2.5. |
| A screen described in this manual is missing | Your account does not have permission for it. See section 12.3. |

## 15.2 Cameras and Video

| Problem | What to check |
| --- | --- |
| Scan finds no cameras | The cameras may be on a different network, or the network may block discovery messages. Add them by hand using section 4.2. |
| A camera shows Offline | Check that it has power and a network connection, and that you can reach its address from this computer. Check the username and password by clicking Test Connection. |
| A camera shows Connecting and never changes | The address is probably reachable but the video port is wrong. Check the RTSP port, usually 554. |
| A camera was working and is now Offline | The system retries by itself. If it does not come back, check the camera and the network between it and this computer. |
| The picture is delayed or jerky | Check the network between the camera and this computer. If detection or face recognition is on for many cameras, turn it off for those that do not need it, or increase Detection Interval in Settings. |
| Everything is slow when several cameras are shown | Show fewer cameras at once, or reduce the cameras running analytics. |

## 15.3 Detection and Face Recognition

| Problem | What to check |
| --- | --- |
| A screen says the model is not loaded | The model file is missing or in the wrong place. Check the path in Settings under AI Models, then click Reload Models; the message says exactly what went wrong. |
| Reload Models says a model was refused | The file is not a model the system can read. Use one of the models named in the README in the Models folder. |
| Detection is on but no boxes ever appear | Check that the camera has Object Detection ticked, that the class you expect is ticked in Detection Settings, and that the confidence threshold is not set too high. |
| Add Face refuses to open | The face models are not loaded. The message names what is missing. |
| No face can be found in a good photograph | Try a clearer, face-on picture. If no picture ever works, the face-finding model may not be supported: open System Information and read the Detector Head and Detector Output rows, and give those to your supplier. |
| Faces are boxed but everyone is Unknown | Register the people again from clearer photographs. If that does not help, lower the face match threshold a little at a time. |
| Almost everyone matches almost everyone, at similar percentages | The recognition model is probably not one the system supports. Open System Information and read the Embedding row, and give it to your supplier. Do not try to fix this by raising the threshold. |

## 15.4 Recording and Playback

| Problem | What to check |
| --- | --- |
| Nothing is being recorded | Check that the camera has Recording ticked and is Online, and that the storage folder exists and can be written to. |
| Recording stopped by itself | The storage limit may have been reached with Overwrite When Full unticked. See section 7.3. |
| A day I expected has no recordings | The retention period may already have removed them, or recording was not on at the time. |
| A recording will not play | The file may have been removed from disk by the retention rules while its entry remained. Check the storage folder. |

## 15.5 Events

| Problem | What to check |
| --- | --- |
| The event log is filling with the same thing | Something stationary is being detected repeatedly. Untick that class in Detection Settings, or aim the camera away from it. |
| The alert bell shows a number that will not clear | Open the Event Log; the count clears when you do. |
| Expected events are missing | Check the filters at the top of the Event Log, including the date range, and clear the search box. |

## 15.6 Before Contacting Support

Open System Information and note the software version, the build date, and the AI Models rows. Then export the recent Event Log entries as described in section 11.3. These two things answer most of the questions a support engineer will ask.

# 16. Reference

## 16.1 Default Account

| Username | Password |
| --- | --- |
| admin | admin1234 |

Important   Change this before the system is put into service. See section 2.5.

## 16.2 Where Things Are Kept

| Item | Location |
| --- | --- |
| Application | The folder you copied it to. |
| Model files | The Models folder inside the application folder. |
| Database | Your Windows profile, under AppData\Local\CameraManagementSystem\cms.db |
| Recordings | The storage folder set in Settings. By default, AppData\Local\CameraManagementSystem\Media |

Note   To back the system up, copy the database file and, if you need the video, the storage folder. Close the application first.

## 16.3 Permissions

| Permission | Gives access to |
| --- | --- |
| Live View | Live View and Camera List |
| Playback | Playback |
| Device Management | Device Management |
| PTZ Control | PTZ Control |
| Object Detection | Object Detection |
| Face Recognition | Face Recognition and Face Database |
| User Management | User Management |
| System Settings | Settings |

Dashboard, Event Log and System Information need no permission and are always available.

## 16.4 Glossary

| Term | Meaning |
| --- | --- |
| ONVIF | A standard that lets cameras from different makers be found and controlled the same way. |
| RTSP | The method by which video is carried from the camera to this computer. |
| PTZ | Pan, tilt and zoom. A camera that can be aimed remotely. |
| Model file | A file containing a trained detection or recognition network. Supplied separately. |
| Threshold | How certain the system must be before it acts on something. |
| Retention | How long recordings are kept before being deleted automatically. |
| Event | A record of something that happened, listed in the Event Log. |
| Group | A label on a registered person: Employee, Visitor, Blacklist or Unknown. |